# ANNEX II + III:	 	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: 	"SUPPLY OF EQUIPMENT FOR PROJECT ERASMUS+ SPACE.COM “NEW STUDY PROGRAM IN SPACE SYSTEMS AND COMMUNICATIONS ENGINEERING"

- Publication reference:	SPACECOM/ GS-LAB

- GENERAL REQUIREMENTS

  - Delivery

All equipment shall be delivered CIP/ DAP Tashkent (INCOTERMS 2010).

  - General requirements applying to all of the items:

    - Unless otherwise explicitly stated in the technical specifications, all technical requirements are minimum.

    - Each of equipment should be fully functional “as a whole”. Proposed hardware products should be supported with full functionality by the related software producers and this issue should be proved by the Participant using the documents of the manufacturers.

    - Each offered item must be stated with brand / product name, product version and/or product/part number.

    - All required components that are not stated explicitly in this Technical Specifications document but are necessary for successful solution to the specifications herein shall be proposed.

    - The type of the supplied current in Uzbekistan is 220 V (mono phase) and 380 V (tri phase + neutral). The quality and stability of the supplied current may undergo fluctuations of ±10 %. All hardware must operate on 220 V  20 V, 50 Hz  0.5 Hz (unless otherwise explicitly stated) power supply and be suitable for direct connection to the standard power outlets in Uzbekistan. The type of electrical outlets generally installed in Uzbekistan is the type with 2 (two) side mounted earthing poles. Electrical plugs of equipment should be compliant with the standards of use in Uzbekistan and fit exactly.

    - The items provided should not be hybrids and should be a registered brand name.

    - All the equipment shall be provided complete with the necessary standard accessories and/or parts such as to ensure that the unit is capable of operating. All specifications details listed for each item is the minimum requirements. Any improvements on the specifications or additional features offered should be clearly identified in the Participant’s offer.

    - The Participants are required to demonstrate that specifications of the items that they offer are responsive to the requirement set out in the bid dossier and to identify model and manufacturer of each individual item in their technical offer and to provide necessary documentation such as catalogues, brochures, booklets that inform of details of technical specifications that sufficiently define the equipments being offered, and thus should enable the contracting authority to check the information provided in the technical offer.

    - All items supplied conforming to the necessary CE regulation / norm must carry a suitable CE badge of conformity, permanently fixed to the machine, where applicable.

  - Technical Documents to be provided

    - Each set of equipment shall be provided with an original set of technical manuals for all parts of the equipment, including all standard accessories, as applicable.

    - The Participant shall with his offer provide brochures, leaflets and other documentation specifying the offered supply in an unambiguous manner.

  - Warranty Condition:

    - The Contractor shall warrant that the supplies are new, unused, of the most recent models and incorporate all recent improvements in design and materials. The Contractor shall further warrant that none of the supplies have any defect arising from design, materials or workmanship. Minimum required warranty period for equipment is indicated in technical specifications to each Item.

    - All repairs and maintenance conducted under the warranty (warranties) must be undertaken by service personnel authorised by the relevant Manufacturer and\or distributor for these tasks.

    - The Participant shall guarantee the proper operation of the Supplies. In the event of failure of the Supplies' operability during the Warranty Period, the Contractor shall restore it, and in case of impossibility to do so, shall replace it with equivalent Supplies in terms of the technical characteristics. The replacement shall be performed by the Contractor at his own expense.

    - The Participant shall replace or restore the Supplies within the scope of the warranty policy.

Column 1-3 should be completed by the Contracting Authority

Column 4-5 should be completed by the Participant

Annex III - the Contractor's technical offer

The Participants are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required parameter (not to be modified by the Participant),

- Column 3 is completed by the Contracting Authority shows the required value of parameter (not to be modified by the Participant),

- Column 4 is to be filled in by the Participant and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 5 allows the Participant to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

EQUIPMENT FOR GROUND STATION LAB

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 1. | HF/VHF/UHF Allmode Transceiver with D-Star DV-Mode |  |  |  |
|  | Intuitive Touch Screen Interface | 48.6 mm x 75.9 mm |  |  |
|  | Multi-band, Multi-mode | HF/50/70/144/430MHz |  |  |
|  | DSP | A high-performance 32-bit floating point IF DSP, which can provide rich digital signal processing features, including digital IF filter, digital twin PBT, noise reduction, CW auto tune, etc. |  |  |
|  | AGC Function Loop | DSP Controlled |  |  |
|  | D-STAR DV Mode | Enabled (Digital Voice + Data) |  |  |
|  | DR (D-STAR Repeater) Mode | Enabled |  |  |
|  | SD Memory Card Slot | Enabled (for Saving Data) |  |  |
|  | RTTY Functions | Built-in |  |  |
|  | CW | full break-in, receive reverse, auto tuning |  |  |
|  | RF speech compressor | controlled by the DSP |  |  |
|  | Repeater channels | 495 regular, 4 call, 6 scan edge and 900 DR mode |  |  |
|  | TX voice memories | 4 channels |  |  |
|  | Frequency stability | ±0.5ppm |  |  |
|  | IF output for DRM | 12kHz |  |  |
|  | Straight Forward Operation | Yes |  |  |
|  | Software Keypad | Yes |  |  |
|  | Separate control panel | Yes |  |  |
|  | Speaker and Jacks | Controller Mounted |  |  |
|  | Interfaces cables | Standard set included |  |  |
|  | Warranty period | 1 year |  |  |
|  | Documents to be provided together with technical offer | - Copy of Manufacturer’s international ISO 9001 and 14001 - Authorisation letter from the Manufacturer/ Authorized distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document from the the Manufacturer/ Authorized distributor/ Authorized Service center proving that the equipment has authorised service on the territory of Uzbekistan |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 2. | Power Supply to HF/VHF/UHF Transceiver |  |  |  |
|  | Switching power supply unit | External |  |  |
|  | Input voltage: | 240V/ 50Hz |  |  |
|  | Ripple and noise level | Not more than 3mV |  |  |
|  | Type | Transformer |  |  |
|  | Warranty | - 1 year |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 3. | Azimuth/Elevation Combination Rotator |  |  |  |
|  | Wind Load | 1,0 m² |  |  |
|  | K-Factor | 578 ft.-lbs |  |  |
|  | Stationary Torque AZ | 40 kg/m |  |  |
|  | Stationary Torque EL | 40 kg/m |  |  |
|  | Rotation Torque AZ | 6 kg/m |  |  |
|  | Rotation Torque EL | 14 kg/m |  |  |
|  | Max Vertical Load | 200 kg |  |  |
|  | Pointing accuracy | not more than ±4% |  |  |
|  | Mast Size | ø 38-63 mm |  |  |
|  | 360º Rotation Time AZ | not more than 58sec @ 50Hz |  |  |
|  | 180º Elevation Time EL | not more than 67sec @ 50Hz |  |  |
|  | Boom Diameter EL | ø 32-43 |  |  |
|  | Weight | Not more than 9 kg |  |  |
|  | Cable Requirement | 2 x 6 conductors |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 4. | Mast preamplifier |  |  |  |
|  | Freq Range MHz (MHz) | 430 – 440 |  |  |
|  | Insertion Loss (dB) | 0.15 |  |  |
|  | Noise Figure (dB) | 0.7 |  |  |
|  | Amplification (dB) | typ. 12 – 22 |  |  |
|  | Max. Power | - Mit Sequencer: 500 (SSB), 300 (FM) - mit VOx: 50 W FM |  |  |
|  | Current Intake (A) | 0.320 |  |  |
|  | Max. Power Handling | 500W |  |  |
|  | Noise Figure | 0.8 |  |  |
|  | Connector A | N Socket |  |  |
|  | Connector B | N Socket |  |  |
|  | Supported Bands | 70cm |  |  |
|  | Max. Mast Diameter (mm) | 58 |  |  |
|  | Features | Adjustable gain, low noise and good large-signal response |  |  |
|  | Built | with a GaAs MMIC of the latest technology on high quality microwave substrate in SMD technology |  |  |
|  | Housing | UV-resistant, plastic |  |  |
|  | Must include | Galvanized mast clamps with stainless steel screws |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 5. | X-Quad Antenna |  |  |  |
|  | Switchable polarisation | possible (hor, vert, circ. right, circ. left, diagonal) |  |  |
|  | Size | Short boom length and compact overall size |  |  |
|  | Frequency | 432 MHz |  |  |
|  | Mounting possibilities | in front of mast or centrally on mast |  |  |
|  | Stacking Distance | 1,1m |  |  |
|  | Max. power load | 1000W |  |  |
|  | Elements per polarisation plane | 18 |  |  |
|  | Stacking Distance (cm) | 1,1m |  |  |
|  | Front/Back ratio (dB) | 21 |  |  |
|  | 3 dB angle horizontal (E) | 36 |  |  |
|  | Supported Bands | 70cm |  |  |
|  | Length [m] | 12.7 m |  |  |
|  | Height [m] | 22 cm |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 6. | Lightning protection |  |  |  |
|  | Max. Power 500-3000 MHz (W) | 400 |  |  |
|  | Insertion Loss 500 MHz (dB) | < 0.3 |  |  |
|  | Insertion Loss 1500 MHz (dB) | < 0.3 |  |  |
|  | Insertion Loss 3000 MHz (dB) | < 0.3 |  |  |
|  | Upper frequency limit (kHz) | 3000 |  |  |
|  | Max. Power 30-500 MHz (W) | 400 |  |  |
|  | Max. Power 0-30 MHz (W) | 400 |  |  |
|  | Breakthrough Voltage (V) | 1000 |  |  |
|  | Max. Power Handling | 400W |  |  |
|  | Connector B | N Socket |  |  |
|  | Connector A | N Plug |  |  |
|  | Use | in-line |  |  |
|  | Can be used | in the coax cable directly at the antenna base |  |  |
|  | M4 thread | Yes, for connection of a grounding wire |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 7. | Coaxial Cable |  |  |  |
|  | Features | Highly flexible, low-loss coaxial cable |  |  |
|  | Length | 40m |  |  |
|  | Use | up to 8 GHz, up to the microwave range |  |  |
|  | Dielectric | Low-loss PE-LLC |  |  |
|  | Gas content | of more than 70% |  |  |
|  | Inner conductor | 7-wire hybrid, with aluminum core and welded copper sheath |  |  |
|  | Shielding | Double |  |  |
|  | N connector | User-friendly solderless |  |  |
|  | Additional features | Low attenuation, ultra-flexible, anti-radiation safe |  |  |
|  | Diameter | 10.2 ± 0.2 mm |  |  |
|  | Impedance | 50 ± 2 Ω |  |  |
|  | Attenuation at 1 GHz / 100 m | 13.49 dB |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 8. | 4-way Coax switch |  |  |  |
|  | 4 fold | 2xN 2xPL |  |  |
|  | Excellent frequency response | up to 600MHz (CO-201) |  |  |
|  | Max. Power Handling | 1500W |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 9. | Spectrum Analyzer |  |  |  |
|  | Frequency range | 5 kHz to 3 GHz |  |  |
|  | Frequency resolution | 1Hz |  |  |
|  | RF input | - Impedance: 50 Ω - Connector: N female - Input attenuator: 0 dB to 40 dB in 5 dB steps - VSWR<2dB |  |  |
|  | RF output | - Impedance: 50 Ω - Connector: N female - VSWR: <2dB |  |  |
|  | Tracking generator must be included | - Internal VSWR bridge - Independent signal source |  |  |
|  | Frequency readout | - Marker resolution: 0.1 Hz - Number of sweep (trace) points: 1183 - Marker tuning frequency step size: span/1182 - Frequency counter resolution: 0.1 Hz - Frequency span: 0 Hz, 10 Hz to 3 GHz - Span uncertainty: 1% |  |  |
|  | Sweep time | - span = 0 Hz: 100 µs to 100 s - 10 Hz ≤ span ≤ 600 MHz: 20 ms to 1000 s - span > 600 MHz: 20 ms × span/600 MHz to 1000 s |  |  |
|  | Level display | - Logarithmic level axis: 1/2/5/10/20/50/100 dB, 10 divisions - Linear level axis: 0 % to 100 %, 10 divisions: - Trace detectors: max. peak, min. peak, auto peak, sample, RMS - Trace functions: clear/write, max. hold, min. hold, average, view - Setting range of reference level: -–130 dBm to +30 dBm - Units of level axis: dBm, dBmV, dBµV, V, W |  |  |
|  | Trigger functions | Trigger source: free run, video, external, IQ power (ASK/FSK only) |  |  |
|  | Resolution bandwidth | 1 Hz to 3 MHz |  |  |
|  | Video filters Range | 1 Hz to 3 MHz |  |  |
|  | Displayed average noise level (preamplifier on) | - < –147 dBm, typ. –157 dBm (1 MHz to 10 MHz) - < –158 dBm, typ. –165 dBm (10 MHz to 2 GHz) - < –155 dBm, typ. –163 dBm (2 GHz to 3 GHz) |  |  |
|  | Intermodulation-free dynamic range (signal level 2 × –20 dBm, RF attenuation = 0 dB) | - fin = 1 GHz - +7 dBm (meas.) - fin = 2.4 GHz - +10 dBm (meas.) |  |  |
|  | Spectral purity, SSB phase noise | - 30 kHz/ < –88 dBc (1 Hz), typ. –92 dBc (1 Hz) - 100 kHz/ < –98 dBc (1 Hz), typ. –103 dBc (1 Hz) - 1 MHz/ < –120 dBc (1 Hz), typ. –125 dBc (1 Hz) |  |  |
|  | Total measurement uncertainty | < 1.25 dB, 0.5 dB (typ.) |  |  |
|  | Level | - Maximum input level (RF input): 33 dBm (= 2 W) - DC voltage: 50 V |  |  |
|  | Modulation Analysis for AM, FM, ASK, FSK | carrier power, carrier frequency offset, AM modulation depth, FM deviation, modulation frequency, THD, SINAD |  |  |
|  | Display type | 10.1" WXGA (1366 × 768 pixel) |  |  |
|  | Remote control | - LAN interface, USB - WiFi (LAN 802.11 b/g/n, 2.4 GHz) to control analyzer by smartphone (android, iOS): mandatory |  |  |
|  | Additional measurements | - Vector Network Analyzer Measurements must be included - S11, S21 parameters measurements - 1-port cable loss measurement - Distance-to-fault analysis (return loss, VSWR with average and maximum indication) |  |  |
|  | Spectrum Analyzer Preamplifier | included |  |  |
|  | Receiver mode functionality | included |  |  |
|  | Measurements | - fixed frequency - channel scan - channel scan (user defined channel list) - EMI precompliance ( CISPR bandwidth, CISPR detectors) |  |  |
|  | Advanced measurements functions | - Spectrogram - Channel power - Ocupied bandwidth - Third-order intercept - Harmonic distortion - TDMA power - AM modulation depth |  |  |
|  | Data storage | - Internal: > 256 instrument settings and traces - on memory stick, ≥ 1 Gbyte: > 5000 instrument settings and traces |  |  |
|  | Power consumption | less than 16 W |  |  |
|  | Weight | less than 3.5 kg |  |  |
|  | User interface | Ergonomic |  |  |
|  | Must include | All standard interface cables and connectors |  |  |
|  | Warranty period | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Copies of Manufacturer’s international ISO 9001, ISO 14001 and ISO 17025 certificates - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document from the Manufacturer/ Authorised distributor proving that the equipment has authorised service on the territory of Uzbekistan |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 10. | LCR-Bridge |  |  |  |
|  | Test frequency | 20 Hz to 200 kHz (69 steps) |  |  |
|  | Transformer Parameter Measurement and Internal programmable Voltage and Current Bias | - Internal Bias voltage: 0…+ 5.00Vdc - External Bias voltage: 0…+40Vdc (fused 0.5A |  |  |
|  | Trigger | - Continuous - Manual or external via interface - Trigger Input |  |  |
|  | Measurement functions | L, C, R, ¦Z¦, X, ¦Y¦, G, B, D, Q, φ, Δ, M, N |  |  |
|  | Measurement modes | Auto, L+Q, L+R, C+D, C+R, R+Q, Z+Q, Y+Q, R+X, G+B, N-Q, M |  |  |
|  | Equivalent circuits | - Auto - Series - Parallel |  |  |
|  | Basic accuracy | 0.05 % |  |  |
|  | Limits of measurements | - ¦Z¦, R, X: 0.01mΩ…100MΩ - ¦Y¦, G, B: 0.01µS…1000S - C: 0.01pF…0.1F - L: 0.01µH…100000H - D: 0.0001…9.99 - Q: 0.1…9999.9 - φ: -180°…+180° - ∆: -999.99%...999.99% - M: 1µH…100H - N: 0.95…500 |  |  |
|  | Measurement rate | Up to 12 measurements per second |  |  |
|  | Modes | Parallel, Serial |  |  |
|  | Test signal level monitor | Voltage, Current |  |  |
|  | Error correction | - Open - Short - Load |  |  |
|  | Inputs/Outputs and Interfaces | - H Cur/H pot - L Cur/L pot - External BIAS - Trigger Input - Dual interface USB/RS-232 |  |  |
|  | Cables and connectors | - four-terminal Kelvin test cable - four-terminal SMD component test fixture - power cable |  |  |
|  | Power supply/consumption | - Power supply: 110 V to 230 V ±10 %, 50 to 60 Hz, CAT II - Power consumption: less than 20W |  |  |
|  | Environmental conditions | - Operating temperature: +5 °C to +40 °C - Storage temperature: -20 °C to +70 °C |  |  |
|  | Must include | All standard interface cables and connectors |  |  |
|  | Warranty period | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Copies of Manufacturer’s international ISO 9001, ISO 14001 and ISO 17025 certificates - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document from the Manufacturer/ Authorised distributor proving that the equipment has authorised service on the territory of Uzbekistan |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 11. | Arbitrary Function Generator |  |  |  |
|  | Waveform modes | sine, square, pulse, triangle, ramp and arbitrary waveforms (incl. standard curves: white noise, pink noise, cardiac, exponential rise and fall) |  |  |
|  | Arbitrary Waveform | - Sample Rate: 250 MSa/s - Frequency range: 10 μHz to 12.5 MHz - Signal Rate: 1... 256kPts - Waveform Length: up to 256 kSa - Amplitude Resolution: 14 bits - Internal non-volatile memory: up to 4 MB - Rise and fall times: 8ns - Sweep signals: all - Burst signals:all |  |  |
|  | Frequency Characteristics | Range - Sine, Square: 10 μHz to 25 MHz Accuracy - Temperature stability: 1x 10-6 (+18°C to +28 °C) - Aging (after one year): ±1x 10-6 (+25 °C) - Square accuracy: ±1 % + 5 ns |  |  |
|  | Amplitude | Range: - 5 mVPP to 10 VPP (into 50 Ω) - 10 mVPP to 20 VPP (open circuit) Accuracy: ±1% of setting ±1 mVPP at 1 kHz Resolution: 1 mV Flatness: - up to 10 MHz - ±0.15 dB - 10 MHz to 25 MHz - ±0.2 dB - above 25 MHz - ±0.4 dB Units: Vpp or dBm, selectable |  |  |
|  | Offset | Range: - ±5 mV to 5 V (into 50 Ω) - ±10 mV to 10 V (open circuit) Accuracy: ±2 % of offset setting ±0.5 % of amplitude setting ±2 mV ±1 mV / MHz |  |  |
|  | Output | Impedance: 50Ω Voltage: 5mVpp…10Vpp (into 50Ω) 10mVpp…20Vpp (open circuit) Overload protection: short-circuit-proof, max. ±15 V of external voltage |  |  |
|  | Trigger output | - Level: TTL, positive slope - Impedance: 50Ω |  |  |
|  | Sine wave Characteristics | Harmonic Distortion: - up to 100 kHz - < -70 dBc - 100 kHz to 10 MHz - < -55 dBc - 10 MHz to 25 MHz - < -40 dBc - above 25 MHz - < -37 dBc |  |  |
|  | Square wave Characteristics | - Frequency: 10µHz…25MHz - Rise and fall times: 8 ns, fixed - Overshoot: < 3% (typ.) - Symmetry: duty cycle: 50%, accuracy: ±1 % + 5 ns - Jitter: <1 nsrms (typ.) |  |  |
|  | Pulse Characteristics | - Frequency: 100µHz…12.5MHz - Rise and fall times: 8 ns to 500 ns, variable - Overshoot: < 3% (typ.) - Pulse width: 10ns...999s - Jitter: < 500 psrms (typ.) |  |  |
|  | Ramp and Triangle | - Frequency: 10µHz…5MHz - Linearity: - up to 250 kHz - < 0.1% (typ.) - above 250 kHz - < 2% (typ.) - Ramp symmetry: - 0% to 100%, resolution 0.1% - (0% ≙ negative ramp, - 100% ≙ positive ramp, 50% ≙ triangle) |  |  |
|  | Waveform modulation | AM, FM, PM, PWM, FSK |  |  |
|  | Save and recall | on internal file system (up to 4 MB) or external USB memory (max. 4 GB) |  |  |
|  | Generator must include | External SW for PC installation for the following functions: - Generating a different waveforms (Arbitrary): Trapezoid, Spectrum (inverse FFT), Noise, Square, Formula, Pulse, Digital - Import signals created in *.csv files - Summing of different signals - Generate the product of previously generated arbitrary waveforms - Edit signals - Make screenshots - Control of generator |  |  |
|  | Included interfaces | at least 1x USB-host (type A) and Ethernet (RJ45) / USB-device (type B) |  |  |
|  | Display | 3.5” color TFT with at least 320 x 240 resolution and LED backlit |  |  |
|  | Power supply/consumption | - AC Power supply: 105 V to 253 V, 50 Hz to 60 Hz, CAT II - Power Consumption: 30 W |  |  |
|  | Environmental conditions | - Operating temperature range: +5°C to +40 °C - Storage temperature range: -20°C to +70 °C - Relative Humidity: 5% to 80 % (without condensation) |  |  |
|  | Must include | All standard interface cables and connectors |  |  |
|  | Warranty period | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Copies of Manufacturer’s international ISO 9001, ISO 14001 and ISO 17025 certificates - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document from the Manufacturer/ Authorised distributor proving that the equipment has authorised service on the territory of Uzbekistan |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 12. | Phase line |  |  |  |
|  | Must combine | Two radiators to one cable |  |  |
|  | Application | For circular polarization |  |  |
|  | Type | 2m f. X-Quad w. connectors |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 13. | Power splitter |  |  |  |
|  | Max. Power Handling | 2000W |  |  |
|  | Supported Bands | 2m |  |  |
|  | Type | 2 m f. 2 antennas |  |  |