# ANNEX II + III:	 	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: 	"SUPPLY OF EQUIPMENT FOR PROJECT ERASMUS+ SPACE.COM “NEW STUDY PROGRAM IN SPACE SYSTEMS AND COMMUNICATIONS ENGINEERING"

- Publication reference:	SPACECOM/2021

- GENERAL REQUIREMENTS

  - Delivery

All equipment shall be delivered CIP/ DAP Tashkent (INCOTERMS 2010).

  - General requirements applying to all of the items:

    - Unless otherwise explicitly stated in the technical specifications, all technical requirements are minimum.

    - Each of equipment should be fully functional “as a whole”. Proposed hardware products should be supported with full functionality by the related software producers and this issue should be proved by the Participant using the documents of the manufacturers.

    - Each offered item must be stated with brand / product name, product version and/or product/part number.

    - All required components that are not stated explicitly in this Technical Specifications document but are necessary for successful solution to the specifications herein shall be proposed.

    - The type of the supplied current in Uzbekistan is 220 V (mono phase) and 380 V (tri phase + neutral). The quality and stability of the supplied current may undergo fluctuations of ±10 %. All hardware must operate on 220 V  20 V, 50 Hz  0.5 Hz (unless otherwise explicitly stated) power supply and be suitable for direct connection to the standard power outlets in Uzbekistan. The type of electrical outlets generally installed in Uzbekistan is the type with 2 (two) side mounted earthing poles. Electrical plugs of equipment should be compliant with the standards of use in Uzbekistan and fit exactly.

    - The items provided should not be hybrids and should be a registered brand name.

    - All the equipment shall be provided complete with the necessary standard accessories and/or parts such as to ensure that the unit is capable of operating. All specifications details listed for each item is the minimum requirements. Any improvements on the specifications or additional features offered should be clearly identified in the Participant’s offer.

    - The Participants are required to demonstrate that specifications of the items that they offer are responsive to the requirement set out in the bid dossier and to identify model and manufacturer of each individual item in their technical offer and to provide necessary documentation such as catalogues, brochures, booklets that inform of details of technical specifications that sufficiently define the equipments being offered, and thus should enable the contracting authority to check the information provided in the technical offer.

    - All items supplied conforming to the necessary CE regulation / norm must carry a suitable CE badge of conformity, permanently fixed to the machine, where applicable.

  - Technical Documents to be provided

    - Each set of equipment shall be provided with an original set of technical manuals for all parts of the equipment, including all standard accessories, as applicable.

    - The Participant shall with his offer provide brochures, leaflets and other documentation specifying the offered supply in an unambiguous manner.

  - Warranty Condition:

    - The Contractor shall warrant that the supplies are new, unused, of the most recent models and incorporate all recent improvements in design and materials. The Contractor shall further warrant that none of the supplies have any defect arising from design, materials or workmanship. Minimum required warranty period for equipment is indicated in technical specifications to each Item.

    - All repairs and maintenance conducted under the warranty (warranties) must be undertaken by service personnel authorised by the relevant Manufacturer and\or distributor for these tasks.

    - The Participant shall guarantee the proper operation of the Supplies. In the event of failure of the Supplies' operability during the Warranty Period, the Contractor shall restore it, and in case of impossibility to do so, shall replace it with equivalent Supplies in terms of the technical characteristics. The replacement shall be performed by the Contractor at his own expense.

    - The Participant shall replace or restore the Supplies within the scope of the warranty policy.

Column 1-3 should be completed by the Contracting Authority

Column 4-5 should be completed by the Participant

Annex III - the Contractor's technical offer

The Participants are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required parameter (not to be modified by the Participant),

- Column 3 is completed by the Contracting Authority shows the required value of parameter (not to be modified by the Participant),

- Column 4 is to be filled in by the Participant and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 5 allows the Participant to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

COMPUTER EQUIPMENT

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 1. | Workstation PC Type 1 |  |  |  |
|  | CPU | Intel Xeon W-1250 |  |  |
|  | RAM | 16GB DDR4 3200 |  |  |
|  | Video | NVIDIA Quadro RTX 4000 8GB PCIe x16 |  |  |
|  | Storage | 1TB 2280 PCIe-3x4 NVMe SED |  |  |
|  | Networking | Gigabit Ethernet |  |  |
|  | PSU | 500W |  |  |
|  | Keyboard and Mouse | Wired USB |  |  |
|  | Warranty | 3 years onsite service |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 2. | 27-inch monitor |  |  |  |
|  | Display size | 27 inches |  |  |
|  | Native resolution | Full HD (1920*1080) |  |  |
|  | Matrix type | IPS |  |  |
|  | Video connections | one HDMI and one DisplayPort |  |  |
|  | Warranty period | - 1 year |  |  |
|  | Documents to be provided together with technical offer | - Document from the Manufacturer/ Authorised distributor proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 3. | Stereo Monitor 3D |  |  |  |
|  | Diagonal | 21.5 inches |  |  |
|  | 3D technology | Linear polarization |  |  |
|  | Viewing angle horizontal / vertical | 170°/ 160° |  |  |
|  | Response time | 2ms |  |  |
|  | Contrast ratio | 200,000:1 ACR |  |  |
|  | Integrated speakers | 2x2 W |  |  |
|  | Connection | 2x DisplayPort |  |  |
|  | Scope of delivery | 3x polarizing glasses, power connection cable |  |  |
|  | Warranty | 12 months |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 4. | GNSS Receiver |  |  |  |
|  | Set must include | Base GPS/GNSS receiver with full set of required options, Rover GPS/GNSS receiver with full set of required options, Controller with software |  |  |
|  | Number of channels | 800 |  |  |
|  | GNSS signals | GPS, GLONASS, BEIDOU, GALILEO, QZSS and IRNSS |  |  |
|  | Radio | 2-5W UHF radio with 410-470MHz frequency, Tx- Rx |  |  |
|  | Radio antenna | External |  |  |
|  | Display | Colour touch |  |  |
|  | Hot start | <15s |  |  |
|  | Internal memory | 32GB |  |  |
|  | Position rate | 10Hz |  |  |
|  | Accuracy | Code Differential positioning: 0.40 m RMS SBAS positioning depending on SBAS system performance: 0.60 m RMS |  |  |
|  | RTK signal initialization | 2 to 8 seconds |  |  |
|  | Battery | Internal, Rechargeable, 16.300 mAh |  |  |
|  | Communications | 5 pins Lemo, 1 PPS port, GNSS port for external antenna, Bluetooth 2.1.+ EDR V 4.1., Wi-Fi b/g/n, Web UI |  |  |
|  | Outputs | RTCM 3.0, 3.2, CMR, CMR+, DGPS, NMEA 0183 |  |  |
|  | Documents to be provided together with technical offer | - Copy of Manufacturer’s international ISO 9001 - Authorisation letter from the Manufacturer, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document from the Authorized Service center proving that the equipment has authorised service on the territory of Uzbekistan |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 5. | Total Station |  |  |  |
|  | Set must include | Total Station, wooden tripod 1700mm, round prism, telescopic pole 3.15m |  |  |
|  | Long range measurements | 600/1000m in reflectorless mode Up to 5000 m using a single prism |  |  |
|  | Angle measurement: | Accuracy: 2” Reading system: Absolute encoder Display resolution: Selectable. 1”/ 5”/ 10” 0.0001g/ 0.0005g/ 0.001g 0.01mil/ 0.05 mil/ 0.1mil Angle units: DEG 360°/ GON 400/ MIL 6.400 |  |  |
|  | Telescope | Magnification/ Field of view: 30x/ 1°30’ Tube length: 156mm Minimum focus distance: 1.0m Reticle: ON/ OFF Objective aperture: φ45mm Laser pointer: Red light, coaxial |  |  |
|  | Tilt sensor | Type: Electronic, dual-axis Compensation range/ accuracy: ±3.0’/ 1” |  |  |
|  | Distance measurements accuracy | Standard mode prism: 2mm + 2ppm Long mode prism: 2mm + 2.5ppm Reflective sheet (6cm x 6 cm): 3mm + 2ppm Reflectorless: 3mm + 2ppm |  |  |
|  | Measurement time | Standard mode/ prism (Tracking/ Fast/ Fine): 0.4/ 0.6/ 1/0 sec/ Reflectorless: 1.5÷5 sec |  |  |
|  | Distance measurement | Distance unit: m/ US ft/ INT ft Display resolution: 0.001m, 0.01ft |  |  |
|  | Laser Plummet | Laser type: 635nm semiconductor Accuracy: 1mm/ 1.5m Spot: ±1.5mm/ 1.5m |  |  |
|  | Level Vial Sensitivity | Plate level: 30”/ 2mm Circular level: 8’/ 2mm |  |  |
|  | Waterproof/ Dustproof | IP55 |  |  |
|  | Specifications | Display/ keyboard: Two sided, LCD 240x128, Dot/ Alphanumeric Memory: 4Gb internal, SD card (max 16Gb) Interface: RS-232C/ mini USB/ SD card/ Bluetooth Sensor: Temperature/ Pressure |  |  |
|  | On Board Field Application Programs | Data recording and management, Stake out, Area & Volume, Target offset, Hidden point, COGO, Tie distance, Remote height, Height transfer, Free station, Line/Arc stake out, Construction, Lead measure, Road |  |  |
|  | Warranty | 2 years |  |  |
|  | Documents to be provided together with technical offer | - Copy of Manufacturer’s international ISO 9001 - Authorisation letter from the Manufacturer, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document from the Authorized Service center proving that the equipment has authorised service on the territory of Uzbekistan |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 6. | Professional display |  |  |  |
|  | Diagonal | 64.5", 164cm |  |  |
|  | Panel | IPS |  |  |
|  | Native resolution | 3840 x 2160 @60Hz (8.3 MegaPixels 4K UHD) |  |  |
|  | Static contrast | 1300:1 typical |  |  |
|  | Viewing zone | horizontal/vertical: 178°/178° |  |  |
|  | Interfaces/ Connectors | VGA x1, DVI x1, HDMI x2, DisplayPort x1, Mini jack x1, RJ45 (LAN) x1, IR x1 (3.5mm stereo mini), S/PDIF (Optical) x1, Speakers 2 x 10W, USB ports x2 |  |  |
|  | Set must include | Power, HDMI, RS-232c cables, Remote control |  |  |
|  | Warranty period | 1 year |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 7. | Workstation PC type 2 |  |  |  |
|  | CPU | Intel Core i7 10700 |  |  |
|  | RAM | 16GB (2x8GB) DDR4 2933 |  |  |
|  | Video | NVIDIA GeForce RTX2060 Super 8GB |  |  |
|  | Storage | 512GB M.2 2280 PCIe NVMe TLC SSD, 2TB 7200RPM SATA 3.5in |  |  |
|  | Networking | Gigabit Ethernet |  |  |
|  | PSU | 550W |  |  |
|  | Keyboard and Mouse | Wired USB |  |  |
|  | Warranty | 3 years onsite service |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 8. | Laptop PC Type 1 |  |  |  |
|  | CPU | Intel Core i5-1135G7 |  |  |
|  | RAM | 4 GB DDR4 3200 |  |  |
|  | Storage | 256 GB PCIe NVMe M.2 SSD |  |  |
|  | Screen | 13.3" diagonal, FHD, 250 nits |  |  |
|  | Webcam | HP 720p |  |  |
|  | Connectivity | Wi-Fi 6 ax, Bluetooth 5, |  |  |
|  | Warranty | 1 year |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 9. | Laser multifunctional unit |  |  |  |
|  | Functions | Print, Copy, Scan, Fax, Email |  |  |
|  | Print speed, black | Up to 40 ppm |  |  |
|  | Print Technology | Laser |  |  |
|  | Display | 2.7" color touchscreen |  |  |
|  | Paper handling | 100-sheet tray 1, 250-sheet input tray 2; 50-sheet ADF |  |  |
|  | Connectivity | Hi-Speed USB 2.0; 1 Front USB port; Gigabit Ethernet; Wi-Fi |  |  |
|  | Duplex printing | Yes, automatic |  |  |
|  | Memory size | 512MB |  |  |
|  | Warranty | 1 year |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 10. | Wide-format inkjet multifunctional unit |  |  |  |
|  | AIO functions | Wireless, Print, Fax, Scan and Copy |  |  |
|  | Print Speed | Black: Up to 22 ppm (ISO, laser-comparable) Colour: Up to 18 ppm (ISO, laser-comparable) |  |  |
|  | Resolution | Black (best): Up to 1200 x 1200 dpi Colour (best): Up to 4800 by 1200 optimised dpi |  |  |
|  | Wireless capability | built-in WiFi |  |  |
|  | Mobile printing capability | Apple AirPrint™, HP ePrint, Wireless direct printing, Mopria-certified and Google Cloud Print™ |  |  |
|  | Copy Speed | Black: Up to 18 cpm (ISO, laser-comparable) Colour: Up to 13 cpm (ISO, laser-comparable) |  |  |
|  | Duplex print options | Automatic (default) |  |  |
|  | Supported sizes | Up to A3 |  |  |
|  | Paper handling | 35-sheet ADF, 250-sheet input tray, 75-sheet output tray |  |  |
|  | Connectivity | 1 USB 2.0; 1 Ethernet; 1 Wireless 802.11b/g/n; 2 RJ-11 modem ports |  |  |
|  | Memory | 512MB |  |  |
|  | Warranty | 1 year |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 11. | FDM 3D printer |  |  |  |
|  | Material support | PLA,PETG,SILK |  |  |
|  | Max printable area | 300*300*300mm |  |  |
|  | Filament diameter/light | 1.75MM |  |  |
|  | Layer thickness | 100-400 um |  |  |
|  | LCD display size | 75*51 mm |  |  |
|  | Extruder quantity | 1 |  |  |
|  | Plateform | Aluminium plateform W/ heating |  |  |
|  | Warranty | 1 year |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 12. | Workstation PC type 3 |  |  |  |
|  | CPU | Intel Xeon W-1250 |  |  |
|  | RAM | 128GB (4x32GB) DDR4 3200 |  |  |
|  | Video | NVIDIA Qdr RTX 6000 24GB |  |  |
|  | Storage | 2x 1TB 7200 SATA 3.5in |  |  |
|  | PSU | 700W |  |  |
|  | Keyboard and Mouse | Wired USB |  |  |
|  | Warranty | 3 years onsite service |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 13. | Graphic Tablet |  |  |  |
|  | Number of pressure levels | 4 096 |  |  |
|  | Size | 264 × 200 × 8.8mm |  |  |
|  | Work surface resolution (lines per inch) | 2 540 |  |  |
|  | Pen reading speed | 133pps |  |  |
|  | Number of ExpessKeys | 4 |  |  |
|  | Integrated Bluetooth | Yes |  |  |
|  | Warranty | 1 year |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 14. | LCR-Bridge |  |  |  |
|  | Display | Measurement modes: Auto, L-Q, L-R, C-D, C-R, R-Q, Z-Θ, Y-Θ, R-X, G-B, N-Θ, M Equivalent circuits: auto, series or parallel Parameters displayed: Value, deviation or % deviation Averaging: 2 to 99 measurements |  |  |
|  | Accuracy | Primary parameters: Basic accuracy (Test voltage: 1.0 V, measurement SLOW/MEDIUM, autoranging mode, constant voltage OFF, bias off). For FAST mode double the basic accuracy values Basic accuracy D, Q: ±0,0001 if f = 1 kHz Phase angle: ±0,005° if f = 1 kHz |  |  |
|  | Ranges | ¦Z¦, R, X: 0,01 mΩ to 100 MΩ ¦Y¦, G, B: 10 nS to 1.000 S C: 0,01 pF to 100 mF L: 10 nH to 100 kH D: 0,0001 to 9,9999 Q: 0,1 to 9.999,9 Θ: -180° to +180 ° Δ: -999,99 to 999,99 % M: 1 μH to 100 H N: 0,95 to 500 |  |  |
|  | Measurement conditions and functions | Test frequency: 20 Hz to 200 kHz (69 steps) Frequency accuracy: ±100 ppm AC test signal level: 50 mVrms to 1.5 Vrms Resolution: 10 mVrms Drive level accuracy: ±(5 % + 5 mV) Internal bias voltage: 0 VDC to +5,00 VDC Resolution: 10 mV External bias voltage: 0 VDC to +40 VDC (fused 0.5 A) Internal bias current: 0 mA to +200 mA Resolution: 1 mA Range selection: Auto and Hold Trigger: Continuous, manual or external via interface, binning interface or trigger input Trigger delay time: 0 ms to 999 ms in 1 ms steps Measurement time (f ≥1 kHz): FAST 70 ms, MEDIUM 125 ms, SLOW 0.7 s |  |  |
|  | Miscellaneous | Test signal level monitor: Voltage, current Error correction: Open, short, load Save/Recall: 9 instrument settings Front-end protection: Vmax <√2/C @ Vmax <200 V, C in Farads (1 Joule of stored energy) Low potential and low current guarding: Ground, driven guard or auto (fused) Constant voltage mode (25 Ω source) Temperature effects R, L or C: ±5 ppm/°C |  |  |
|  | Interface | Dual interface USB/RS-232 (R&S®HO820) |  |  |
|  | Safety class I (EN61010-1) | Safety class I (EN61010-1) |  |  |
|  | Power supply | 110 V to 230 V ±10 %, 50 to 60 Hz, CAT II |  |  |
|  | Environment | Operating temperature: +5 °C to +40 °C Storage temperature: -20 °C to +70 °C Rel. Humidity: 5% to 80 % (non condensing) |  |  |
|  | Accessories supplied: | Line cord, operating manual, 4-terminal kelvin test cable, 4-terminal SMD component test fixture |  |  |
|  | Warranty | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 15. | Spectrum Analyzer |  |  |  |
|  | Frequency range | 5 kHz to 3 GHz |  |  |
|  | Frequency resolution | 1 Hz |  |  |
|  | Resolution bandwidth | 1 Hz to 3 MHz in 1/3 sequence |  |  |
|  | Spectral purity, SSB phase noise | f = 500 MHz: 30 kHz: < –88 dBc (1 Hz), typ. –92 dBc (1 Hz) 100 kHz: < –98 dBc (1 Hz), typ. –103 dBc (1 Hz) 1 MHz: < –120 dBc (1 Hz), typ. –125 dBc (1 Hz) |  |  |
|  | Displayed average noise level | 0 dB RF attenuation, termination 50 Ω, RBW = 100 Hz, VBW = 10 Hz, sample detector, log scaling, normalized to 1 Hz preamplifier = off: 1 MHz to 10 MHz < –127 dBm, typ. –135 dBm, 10 MHz to 2 GHz < –142 dBm, typ. –150 dBm? 2 GHz to 3 GHz < –138 dBm, typ. –147 dBm preamplifier = on: 1 MHz to 10 MHz < –147 dBm, typ. –157 dBm, 10 MHz to 2 GHz < –158 dBm, typ. –165 dBm, 2 GHz to 3 GHz < –155 dBm, typ. –163 dBm |  |  |
|  | Third-order intercept (TOI) | intermodulation-free dynamic range, signal level 2 × –20 dBm, RF attenuation = 0 dB, RF preamplifier = off: fin = 1 GHz +7 dBm (meas.), fin = 2.4 GHz +10 dBm (meas.) |  |  |
|  | One-port vector network analyzer | frequency range: 2 MHz to 1/2/3 GHz output power: –10 dBm |  |  |
|  | Tracking generator | frequency range: 5 kHz to 1/2/3 GHz output power: –30 dBm to 0 dBm Internal VSWR bridge Independent signal source |  |  |
|  | Independent source | frequency range: 5 kHz to 1/2/3 GHz output power: –30 dBm to 0 dBm |  |  |
|  | Included functions | Preamplifier, Modulation analysis for AM, FM, ASK, FSK: carrier power, carrier frequency offset, AM modulation depth, FM deviation, modulation frequency, THD, SINAD , Vector reflection measurements: S11, S21 parameters measurements, 1-port cable loss measurement, Distance-to-fault analysis (return loss, VSWR with average and maximum indication, Receiver mode: fixed frequency, channel scanchannel scan (user defined channel list), EMI precomp liance ( CISPR bandwidth, CISPR detectors), Advanced measurements: Spectrogram, Channel power, Ocupied bandwidth, Third-order intercept, Harmonic distortion, TDMA power, AM modulation depth |  |  |
|  | Included options | 2x RF cable (1m), DC to 8GHz, connectors N(m)/N(f), for DTF measurements 2x Adaptor N/SMA 1x Probe set for E and H nearfield emissions 30 MHz to 3 GHz 1x Preamplifier 100 kHz to 3 GHz, 20 dB power supply 100 - 230 V for probe |  |  |
|  | Set must include | RF Teaching Kit. DUT designed to showcase different RF measurements in a lab environment. . RF teaching kit a universal board with a transceiver, DC/DC converter circuitry for EMI troubleshooting and an onboard calibration kit for network analysis. Allows to demonstrate the functionality of a spectrum analyzer, network analyzer or signal generator. Must have various connectors, jumpers and DIP switches to control its functionality. Jumper: 0: Unused, 1: Jumper for LO frequency selection, 2: Jumper for LO synthesizer state, 3: Jumper for signal routing, 4: Jumper for IF PLL power supply selection, 5: Power supply selection, 6: Calibration standard selection, 7: Attenuator (for up- / downconverter), 8: Baseband state Connectors: A: Power supply (micro USB), B: Power supply ( mini USB), C: Calibration kit (Short / Through), D: Calibration kit (Open / Through), E: Calibration kit (Match) F: SPI interface (internal use for synthesizer, setup), G: PLL supply, AC coupled via 100 μF, H: I/Q input and output (baseband connectors) |  |  |
|  | Warranty | 3 years for main unit and 1 year for accessories |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 16. | Near filed probe set with amplifier for Spectrum analyzer and Oscilloscope |  |  |  |
|  | Set must contain | 1x Probe set for E and H nearfield emissions 30 MHz to 3 GHz 1x Preamplifier 100 kHz to 3 GHz, 20 dB power supply 100 - 230 V for probe HZ-15 |  |  |
|  | Warranty | 1 year |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 17. | Power Supply Type 1 |  |  |  |
|  | Key features | Number of output channels: 3 Max. output power: 110 W Output power per channel: max. 33.6 W Output voltage per channel: 0 V to 32 V Output current per channel: 0 A to 3 A |  |  |
|  | Specialty power supplies | Tailored to specific applications Unique features such as Emulation of the unique characteristics of a battery Electronic loads to accurately sink current and dissipate power in a controlled manner ❙Used in labs and ATE environments |  |  |
|  | Maximum voltage | in serial operation: 96 V in parallel operation: 9 A |  |  |
|  | Voltage ripple and noise | 20 Hz to 20 MHz: typ. < 1.5 mV (RMS) < 20 mV (peak-to-peak) (meas.) |  |  |
|  | Current ripple and noise | 20 Hz to 20 MHz: < 2 mA (RMS) (meas.) |  |  |
|  | Load regulation | load change from 10 % to 90 %: Voltage ± (% of output + offset) < 0.1 % + 20 mV Current ± (% of output + offset) < 0.1 % + 5 mA Load recovery time 10 % to 90 % load change within a band of ± 30 mV of set voltage < 200 μs (meas |  |  |
|  | Line regulation | Voltage ± (% of output + offset) < 0.1 % + 20 mV Current ± (% of output + offset) < 0.1 % + 5 mA Output voltage overshoot at turn-off of mains power and active channel output < 100 mV |  |  |
|  | Programming resolution | Voltage 10 mV Current 1 mA |  |  |
|  | Programming accuracy | Voltage ± (% of output + offset) < 0.1 % + 30 mV Current ± (% of output + offset) < 0.1 % + 5 mA |  |  |
|  | Output measurements | Measurement functions: voltage, current, power Readback resolution: Voltage 10 mV, Current 1 mA Readback accuracy: Voltage ± (% of output + offset) < 0.1 % + 20 mV, Current ± (% of output + offset) < 0.1 % + 5 mA Temperature coefficient +5 °C to +20 °C and +30 °C to +40 °C Voltage ± (% of output + offset) < 0.02 % + 5 mV per K Current ± (% of output + offset) < 0.02 % + 3 mA per K |  |  |
|  | Special functions | Output ramp function (10 ms to 10 s (10 ms increments) Arbitrary function (voltage, current, time, 128 points), 10 ms to 600 s (10 ms increments) Trigger (Digital I/O Minimum trigger interval 10 ms: Trigger response time < 150 ms, Trigger level TTL, max. 5 V, Edge direction rising, falling |  |  |
|  | Protection functions | Overvoltage protection configurable for each channel Overpower protection configurable for each channel Overcurrent protection (electronic fuse) configurable for each channel Response time (Iload > Iresp × 2) < 10 ms Fuse linking (FuseLink function) yes Response time of linked channels < 100 μs + response time of linked channel Fuse delay configurable for each channel 10 ms to 10 s (10 ms increments) Overtemperature protection yes |  |  |
|  | Display and interfaces | 3.5"/QVGA Front panel connections: channel outputs 4 mm safety binding posts Remote control interfaces: standard USB-TMC, USB-CDC (virtual COM) LAN (LXI) Trigger and control interface: digital I/O |  |  |
|  | Warranty | 3 years for main unit and 1 year for accessories |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 18. | Power Supply Type 2 |  |  |  |
|  | Key features | Number of output channels: 2 Total output power:120 W Max. output power per channel: 60 W Output voltage per channel: 0 V to 20 V Max. output current per channel: ≤ 6 V: 6 A, > 6 V: 3 A Load recovery time: < 30 μs Max. power and current per channel when used as a load: 60 W, 3 A |  |  |
|  | Easy operation | High-resolution touchscreen Color coding of operating modes QuickArb function EasyRamp function Save and recall instrument settings |  |  |
|  | Specialty power supplies | Tailored to specific applications Unique features such as - Emulation of the unique characteristics of a battery - Electronic loads to accurately sink current and dissipate power in a controlled manner Used in labs and ATE environments |  |  |
|  | Outputs | Maximum voltage in serial operation: 40 V Maximum current in parallel operation: ≤ 6 V output voltage 12 A, > 6 V output voltage 6 A Adjustable output impedance: –50 mΩ to 100 Ω Increments: 1 mΩ Voltage ripple and noise 20 Hz to 20 MHz: < 500 μV (RMS), < 2 mV (peak-to-peak) (meas.) Current ripple and noise 20 Hz to 20 MHz: < 1 mA (RMS) (meas.) Electronic load: yes, both channels |  |  |
|  | Electronic load | Maximum sink power: 120 W (60 W per channel) Maximum sink current per channel: 3 A Sink modes: constant voltage, constant current, constant resistance |  |  |
|  | Load regulation | load change: 10 % to 90 % Voltage ±(% of output + offset): < 0.01 % + 0.5 mV Current ±(% of output + offset): < 0.01 % + 0.1 mA Load recovery time regulation to within ±20 mV of the set voltage: < 30 μs (meas.) |  |  |
|  | Programming resolution | Voltage: 1 mV Current: 0.1 |  |  |
|  | Programming accuracy | Voltage ±(% of the output + offset): < 0.02 % + 3 mV Current ±(% of the output + offset): < 0.05 % + 2 mA |  |  |
|  | Output measurements | Measurement functions: voltage, current, power, energy Readback resolution Voltage 10 μV Current 10 μA Readback accuracy Voltage ±(% of output + offset) < 0.02 % + 2 mV Current ±(% of output + offset) < 0.05 % + 250 μA Temperature coefficient (per °C) +23 °C (–3 °C/+7 °C) Voltage ±(% of output + offset) 0.15 × specification/°C Current ±(% of output + offset) 0.15 × specification/°C Sense function yes, both channels Maximum sense compensation: 2 V |  |  |
|  | Protection functions | Overvoltage protection adjustable: both channels Programming resolution: 1 mV Overpower protection adjustable,: both channels Overcurrent protection (electronic fuse) adjustable: both channels Programming resolution 0.1 mA Response time (Iload > Iresp × 2) at Iload ≥ 2 A < 1.5 ms Fuse linking (FuseLink function) R&S®NGL202 yes Fuse delay at output-on: both channels 0 ms to 10 s (1 ms increments) Fuse delay time: both channels 0 ms to 10 s (1 ms increments) Overtemperature protection: yes |  |  |
|  | Special functions | Output ramp function EasyRamp EasyRamp time 10 ms to 10 s (10 ms increments) Output sequencing Synchronicity typ. < 25 μs Delay per channel: 1 ms to 10 s (1 ms increments) Arbitrary function QuickArb Parameters: voltage, current, time Maximum number of points: 4096 Dwell time: 1 ms to 20 h (1 ms increments) Repetition continuous or burst mode with 1 to 65535 repetitions Trigger manually via the keyboard, via remote control or via optional interface Trigger and control interfaces digital I/O |  |  |
|  | Display and interfaces | Display: TFT 5" 800 × 480 pixels WVGA Touch Front panel connections: 4 mm safety sockets (channels) Rear panel connections: 8-pin connector block per channel Remote control interfaces: standard USB-TMC, USB-CDC (virtual COM port), LAN |  |  |
|  | Warranty | 3 years for main unit and 1 year for accessories |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 19. | Arbitrary Function Generator |  |  |  |
|  | Device Characteristics | 1 channel, frequency range to 50 MHz Waveforms: Standard: sine, square, pulse, ramp, triangle arbitrary waveforms: up to 256 kSa predefined waveforms: sine, square (50 %), ramp (positive/negative), triangle (50 %), noise (white/pink), cardinal sine, exponential (rise/fall) Operation modes: continuous, modulate, sweep, burst Modulation types: AM, FM, PM, FSK, PWM Temperature stability: 1x 10-6 (+18°C to +28 °C) Aging (after one year): ±1x 10-6 (+25 °C) |  |  |
|  | Waveform Characteristics | Sine Frequency range: 10 μHz to 25 MHz Amplitude flatness: up to 10 MHz ±0.15 dB, 10 MHz to 25 MHz ±0.2 dB, above 25 MHz ±0.4 dB Harmonic distortion: up to 100 kHz < -70 dBc, 100 kHz to 10 MHz < -55 dBc, 10 MHz to 25 MHz < -40 dBc, above 25 MHz < -37 dBc Total harmonic distortion (THD): up to 100 kHz, 0.0 4 % (typ.) Non-harmonic spurious: up to 1 MHz < -70 dBc, above 1 MHz < -70 dBc, increasing +6 dB / decade Phase noise (SSB): 10 kHz Offset -115 dBc / Hz (typ.) |  |  |
|  | Square | Frequency range:¸HMF2525 10 μHz to 25 MHz Rise and fall times : 8 ns, fixed Overshoot: < 3% (typ.) Symmetry: duty cycle: 50%, accuracy: ±1 % + 5 ns Jitter: <1 nsrms (typ.) |  |  |
|  | Pulse | Frequency range: 100 μHz to 12.5 MHz Rise and fall times: 8 ns to 500 ns, variable Overshoot < 3% (typ.) Duty cycle: 0.01% to 99.99% Pulse width: min. 15 ns, resolution 5 ns Jitter: < 500 psrms (typ.) |  |  |
|  | Ramp and Triangle | Frequency range: 10 μHz to 5 MHz Ramp symmetry 0% to 100%, resolution 0.1% (0% ≙ negative ramp, 100% ≙ positive ramp, 50% ≙ triangle) Linearity up to 250 kHz < 0.1% (typ.) above 250 kHz < 2% (typ.) |  |  |
|  | Arbitrary | Frequency range: 100 μHz to 12.5 MHz Waveform length: up to 256 kSa Sample rate: 250 MSa/s Amplitude resolution: 14 bits Internal non-volatile memory: up to 4 MB |  |  |
|  | Connectors | External trigger / gate: Connector BNC socket (front panel) Impedance 5 kΩ ¦¦ 100 pF Polarity positive, negative slope Level TTL, protected up to ±30 V Pulse width min. 100 ns Trigger output: Connector BNC socket (front panel) Impedance 50 Ω Level TTL, positive slope Frequency max. 10 MHz Modulation input: Connector BNC socket (rear panel) Impedance 10 kΩ Voltage level max. ±5 V full-scale Bandwidth (-3 dB) DC to 50 kHz (250 kSa/s sampling rate) Frequency reference input: Connector BNC socket (rear panel) Impedance 1 kΩ Frequency range 10 MHz ±100 kHz Level TTL Frequency reference output: Connector BNC socket (rear panel) Impedance 50 Ω |  |  |
|  | Display | screen size / type: 8.9 cm (3.5”) QVGA color TFT resolution: 320 x 240 backlight: LED Real-time clock (RTC): date and time |  |  |
|  | Accessories supplied | Line cord, Operating manual, Software |  |  |
|  | Set must include | Dual Interface (Ethernet/USB) Generator must include external SW for PC installation for the following functions: Generating a different waveforms (Arbitrary): Trapezoid, Spectrum (inverse FFT), Noise, Square, Formula, Pulse, Digital Import signals created in *.csv files Summing of different signals Generate the product of previously generated arbitrary waveforms Edit signals Make screenshots Control of generator |  |  |
|  | Interfaces | for mass storage: 1x USB-host (type A), FAT16/32 for remote control: RS-232 / USB-device (type B) |  |  |
|  | Warranty | 3 years for main unit and 1 year for accessories |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 20. | Digital Oscilloscope Type 1 |  |  |  |
|  | Number of channels | 4 |  |  |
|  | Bandwidth (–3 dB) | 1 GHz |  |  |
|  | Rise time (calculated) | 1.15 ns |  |  |
|  | Input sensitivity | max. bandwidth in all ranges: at 1 MΩ 500 μV/div to 10 V/div at 50 Ω 500 μV/div to 1 V/div |  |  |
|  | DC gain accuracy | offset and position = 0, maximum operating temperature change of ±5 °C after self-alignment: input sensitivity > 5 mV/div ±1 % of full scale input sensitivity ≤ 5 mV/div to ≥ 1 mV/div ±1.5 % of full scale input sensitivity < 1 mV/div ±2.5 % of full scale |  |  |
|  | ADC resolution | 10-bit, up to 16-bit with high resolution decimation |  |  |
|  | Acquisition system | Maximum realtime sampling rate: 2.5 Gsample/s; 5 Gsample/s, interleaved Acquisition memory: 100 Msample (200 Msample, interleaved); 1 Gsample segmented memory |  |  |
|  | Horizontal system | Timebase range: selectable between 0.5 ns/div and 500 s/div |  |  |
|  | Trigger system | Trigger types standard edge, width, video (PAL, NTSC, SECAM, PAL- M, SDTV 576i, HDTV 720p, HDTV 1080i, HDTV 1080p), pattern, line, serial bus |  |  |
|  | Included applications | I2C/SPI Serial Triggering and Decoding, UART/RS-232/RS-422/RS-485 Serial Triggering and Decoding, CAN/LIN Serial Triggering and Decoding, Audio (I2S, LJ, RJ, TDM) Triggering and Decoding, MIL-STD-1553 Serial Triggering and Decoding, ARINC 429 Serial Triggering and Decoding, Spectrum Analysis and Spectrogram, Power Analysis, Arbitrary Waveform and 4-bit Pattern Generator, Mixed Signal Upgrade for non-MSO models, 400 MHz |  |  |
|  | MSO option | Digital channels: 16 (2 logic probes) Sampling rate :2.5 Gsample/s; 5 Gsample/s, interleaved Acquisition memory: 10 Msample |  |  |
|  | Waveform generator | Resolution, sample rate: 14-bit, 250 Msample/s Amplitude: high Z; 50 Ω 20 mV to 10 V (Vpp); 10 mV to 5 V (Vpp) DC offset: high Z; 50 Ω ±5 V; ±2.5 V |  |  |
|  | Screen | 10.1" WXGA TFT color display (1280 × 800 pixel) |  |  |
|  | Interfaces | USB host with MTP, USB device, LAN, powerful web server for remote display and operation |  |  |
|  | Set must include | mixed signal option, ARB Generator, Application Bundle, DIGITAL STIMULUS BOARD |  |  |
|  | DIGITAL STIMULUS BOARD | 16-channel stimulus and a 16-channel parametric measurement unit for versatile DUT testing. 16-channel digital signal acquisition High measurement resolution of 24 bits for level ranges up to ±27 V Sampling rate of up to 5 ksample/s for inputs Specification: 16 output and 16 input channels Level range ±27 V Digital mode 4 extended analog outputs up to 100 mA output current 100 mA programmable for 4 extended channels Educational Kit key performance specifications Generation of Bus: I2C, CAN, LIN, UART, SPI, I2S Audio block: o I2S (BCLK, LRCLK, I2S_SD o Analog output from microphone: MICP, MICN o Analog Output & PWM (Pulse Width Modulation): ANA, PWM SPI Signals: SPI CSL, SPI CLK, SPI Data CAN Bus: CAN_H, CAN_L Rare Events: SIGNAL, TRIGGER (Use to trigger scope on occurrence of rare error on SIGNAL.) I2C Bus: I2C_SDA, I2C_SCL UART or LIN Signal: UART/LIN Generator: at least 10_MHz_Clk Digital Logic Channels: Digital source of PWM and ANA signals, 8bit Educational Kit key features specifications Simulation of the following signals Normal Mic Signal Sawtooth Digital Counting Pattern Setup of Audio Chip Without AGC Pulse With No Error Distorted Sine Wave 115.2kbaud UART Signal Continuous Readback of ACG (Auto Gain Control) ASCII Freq. Switch Synthesis 19.2kbaud LIN Frame with Checksum Errors Sine Wave Sawtooth Digital Counting Pattern Pulse With Narrow Glitch 100/s Pulse With Narrow Glitch 1/s Pulse With Runt 100/s Pulse With Runt 1/s 250kbps CAN Frame With CRC Error Simulated Power Supply Ripple (mV) |  |  |
|  | Warranty | 3 years for main unit and 1 year for accessories |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 21. | Digital Oscilloscope Type 2 |  |  |  |
|  | X-in-1 oscilloscope | Oscilloscope, Logic analyzer, Protocol analyzer, Waveform and pattern generator, Digital voltmeter, Component tester, Frequency analysis mode, Mask test mode |  |  |
|  | Number of oscilloscope channels | 2 |  |  |
|  | Bandwidth in MHz | 300 |  |  |
|  | Maximum sampling rate in Gsample/s | 1/channel, 2 interleaved |  |  |
|  | Maximum memory depth in Msample | 1/channel, 2 interleaved |  |  |
|  | Timebase accuracy in ppm | 50 |  |  |
|  | Vertical bits (ADC) | 8 |  |  |
|  | Minimum input sensitivity | 1 mV/div |  |  |
|  | Display | 6.5", 640 × 480 pixel |  |  |
|  | Update rate | 10 000 waveforms/s |  |  |
|  | MSO | 8 channels, 1 Gsample/s |  |  |
|  | Math | +, –, *, /, FFT (128k points) |  |  |
|  | Applications included | I²C/SPI serial triggering and decoding, UART/RS-232/RS-422/RS-485 serial triggering and decoding, CAN/LIN serial triggering and decoding, Waveform generator |  |  |
|  | Set must include | Mixed signal upgrade for non-MSO models, 300 MHz , ARB Generator, DIGITAL STIMULUS BOARD |  |  |
|  | DIGITAL STIMULUS BOARD | 16-channel stimulus and a 16-channel parametric measurement unit for versatile DUT testing. 16-channel digital signal acquisition High measurement resolution of 24 bits for level ranges up to ±27 V Sampling rate of up to 5 ksample/s for inputs Specification: 16 output and 16 input channels Level range ±27 V Digital mode 4 extended analog outputs up to 100 mA output current 100 mA programmable for 4 extended channels Educational Kit key performance specifications Generation of Bus: I2C, CAN, LIN, UART, SPI, I2S Audio block: o I2S (BCLK, LRCLK, I2S_SD o Analog output from microphone: MICP, MICN o Analog Output & PWM (Pulse Width Modulation): ANA, PWM SPI Signals: SPI CSL, SPI CLK, SPI Data CAN Bus: CAN_H, CAN_L Rare Events: SIGNAL, TRIGGER (Use to trigger scope on occurrence of rare error on SIGNAL.) I2C Bus: I2C_SDA, I2C_SCL UART or LIN Signal: UART/LIN Generator: at least 10_MHz_Clk Digital Logic Channels: Digital source of PWM and ANA signals, 8bit Educational Kit key features specifications Simulation of the following signals Normal Mic Signal Sawtooth Digital Counting Pattern Setup of Audio Chip Without AGC Pulse With No Error Distorted Sine Wave 115.2kbaud UART Signal Continuous Readback of ACG (Auto Gain Control) ASCII Freq. Switch Synthesis 19.2kbaud LIN Frame with Checksum Errors Sine Wave Sawtooth Digital Counting Pattern Pulse With Narrow Glitch 100/s Pulse With Narrow Glitch 1/s Pulse With Runt 100/s Pulse With Runt 1/s 250kbps CAN Frame With CRC Error Simulated Power Supply Ripple (mV) |  |  |
|  | Warranty | 3 years for main unit and 1 year for accessories |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 22. | Digital Multimeter |  |  |  |
|  | Measurement range | DC to 100 kHz |  |  |
|  | Basic DC accuracy | 0.015 % |  |  |
|  | Number of digits | 5¾ digit |  |  |
|  | Resolution | 1μV , 100 nA , 1 mΩ , 1 pF, 1 Hz, 0.1 C |  |  |
|  | Three values displayed on one screen | Measured voltage, measured current, calculated power |  |  |
|  | Key features | True RMS measurement, AC, AC + DC, Simultaneous display of three measurement functions, e.g. DC + AC + statistics, Measurement functions: DCV, DCI, ACV, ACI, frequency, resistance (2 and 4 wire), temperature, capacitance, diode and continuity test, Mathematic functions: limit testing, minimum, maximum, average, offset, DC power, calculation of dB and dBm, Data logging to internal memory or USB stick |  |  |
|  | Engineering lab | Wide frequency range from DC to 100 kHz, Accurate four-wire resistance measurement, Long-term data logging capability, Fanless design |  |  |
|  | Production environment | LXI-compliant Ethernet interface, USB and Ethernet interface, SCPI remote control functionality, LabVIEW drivers available |  |  |
|  | Warranty | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 23. | Signal Generator |  |  |  |
|  | Flexible and universal all-purpose signal generator | Frequency range 9 kHz to 1.1 GHz or 3.2 GHz Typical maximum level of > +17 dBm Analog modulation modes (AM/FM/φM/pulse modulation) integrated as standard Remote control compatibility with other signal generators Multiple language support (nine selectable GUI languages) Integrated overvoltage protection Wear-free electronic attenuator |  |  |
|  | Frequency | Frequency range: 9 kHz to 3.2 GHz Setting time SCPI mode < 5 ms |  |  |
|  | Level | Maximum output power: f = 200 kHz to 3.2 GHz > +13 dBm f ≥ 500 kHz typ. > +17 dBm in overrange mode Level error: f = 200 kHz to 3.2 GHz ALC ON, AUTO mode, T = 18 °C to 33 °C < 0.9 dB Setting time: SCPI mode < 5 ms Back-feed: 1 MHz ≤ f ≤ 1 GHz 50 W 1 GHz ≤ f ≤ 2 GHz 25 W 2 GHz ≤ f ≤ 3.2 GHz 10 W Maximum permissible DC voltage: 50 V |  |  |
|  | Spectral purity | Nonharmonics carrier offset: > 10 kHz, f ≤ 1600 MHz < –60 dBc (typ. < –72 dBc) SSB phase noise: f = 1 GHz carrier offset = 20 kHz 1 Hz measurement bandwidth < –105 dBc (typ. –111 dBc) Wideband noise: f > 1 MHz, level > 5 dBm carrier offset > 10 MHz 1 Hz measurement bandwidth < –138 dBc (typ. < –146 dBc) |  |  |
|  | Supported modulation modes | AM: standard, AM depth 0 % to 100 % FM/φM standard, Maximum FM deviation f > 1.6 GHz 4 MHz, Maximum φM deviation f > 1.6 GHz 40 rad Pulse standard: Rise/fall time < 500 ns (typ. 100 ns), Minimum pulse width with integrated pulse generator 1 μs, ON/OFF ratio > 80 dB |  |  |
|  | Connectivity | Remote control: Ethernet (TCP/IP), USB Peripherals: USB |  |  |
|  | Warranty | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Authorisation letter from the Manufacturer/ Authorised distributor, which proves that the Participant is authorised to represent the interests of the manufacturer in the current Bid procedure - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 24. | Laptop PC Type 2 |  |  |  |
|  | CPU | Intel Core i7-1165G7 |  |  |
|  | RAM | 16GB (2x8GB) DDR4 3200 |  |  |
|  | Maximum supported memory | 64GB DDR4 3200 |  |  |
|  | Storage | 512GB PCIe NVMe SED OPAL2 TLC SSD |  |  |
|  | Screen | 15.6 FullHD AG LED UWVA 250 fHDC IR bent |  |  |
|  | Video adapter | NVIDIA® T500 (4 GB GDDR6 dedicated) |  |  |
|  | Audio | by Bang & Olufsen |  |  |
|  | Webcam | HP 720p |  |  |
|  | Connectivity | Wi-Fi 6 ax, Bluetooth 5, |  |  |
|  | Warranty | 3 years |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 25. | Desktop PC Type 1 |  |  |  |
|  | CPU | Interl Core i7-10700 |  |  |
|  | RAM | 16GB DDR4 2933 |  |  |
|  | Storage | 1TB SSD |  |  |
|  | Networking | Gigabit Ethrernet |  |  |
|  | Optical Drive | DVD-RW |  |  |
|  | Video Adapter | Dedicated with 2GB GDDR5 |  |  |
|  | Keyboard and Mouse | USB Wired |  |  |
|  | Warranty | 1 years onsite service |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 26. | 24-inch monitor |  |  |  |
|  | Display size | 23.8 inches |  |  |
|  | Native resolution | Full HD (1920*1080) |  |  |
|  | Matrix type | IPS |  |  |
|  | Video connections | one VGA, one DisplayPort, one HDMI 1.4 |  |  |
|  | Warranty period | - 1 year |  |  |
|  | Documents to be provided together with technical offer | - Document from the Manufacturer/ Authorised distributor proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |

| 1. Item Number | 2. Parameter | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation |
| --- | --- | --- | --- | --- |
| 27. | Desktop PC Type 2 |  |  |  |
|  | CPU | Intel Core i3- 10100 |  |  |
|  | RAM | 16GB DDR4 2666 |  |  |
|  | Video Adapter | Integrated |  |  |
|  | Storage | 256GB SSD, 1 TB SATA HDD |  |  |
|  | Networking | Gigabit Ethrernet |  |  |
|  | Optical Drive | DVD-RW |  |  |
|  | Keyboard and Mouse | USB Wired |  |  |
|  | Warranty | 1 years onsite service |  |  |
|  | Documents to be provided together with technical offer | - Document proving that the equipment has authorised service on the territory of Uzbekistan - Copy of Manufacturer’s international ISO 9001, 14001 and CE Mark certificates - Copy of IDC/ Gartner report which include the offered Manufacturer |  |  |